FIRST STEPS WITH USB08 Kit

# Introduction

Thanks for choosing CodeWarrior. This document guides you through the installation and first steps with the USB kit for HC08. It does not replace all the documentation provided, but gives you a good starting point.

This example provides an easy starting point to enter USB technology. The kit includes the application software for the PC side, the hardware and the firmware for the microcontroller. You get all source codes (C/C++) of the demo application which enables you to start your own USB development instantly.

# Installation

- Insert the PRO08 CD in your CD-ROM drive.

- Run the Install.exe on the CD-ROM root if 'autorun' is not enabled for your CD-ROM drive

- Click through the welcome messages, license agreement etc.

- Choose full installation or custom installation (if you want to save disk space). With Custom Installation choose at least 'CodeWarrior IDE', 'Utilities' and 'Motorola M68HC08' (about 155 MB)

- Without a license file, the tools are running in demo mode. If you got a diskette with a license software key on it, copy the 'license.dat' file on this diskette to the directory where you have installed the software. There is already a license.dat file, just append the additional information into it. You can request a free 30 days evaluation license: send an email to license@metrowerks.com.

Added on files:

- Insert the USB08 CD in your CD-ROM drive. Double-click on the self-extracting file “USB08.exe”. This one is located on the CD root.

- Verify that the proposed path corresponds to the examples of your installation. If it does not, you have to modify it.

- Click on Unzip.

# Licensing/Registration

Without a license file, the tools (compiler/assembler/linker/...) are running in size limited demo mode.

With the package you may got a license file (license.dat) either on a diskette or by email.

Append the information in the license.dat file you got using a text editor to the license.dat file installed. This license.dat file is located in the 'root' directory where you have installed the software (usually C:\metrowerks\prog). Make sure that you do not overwrite an existing license file. The license file is a simple text file, so you can add additional information into it. The installation program has created a shortcut to the 'License File' so you can easily locate it.

If you do not have a license file, you can request a free 30 days evaluation license by sending an email to license_europe@metrowerks.com.

If you want to evaluate and install the OSEKturbo source base, you can ask for an evaluation key, which is needed to install the software. Please contact osek@metrowerks.com for this.

If you got a registration card, please take the time to fill it out and send it back as indicated on the registration card.

# Windows Demo Application

This is the application running on the host PC, which will communicate with the board.

The windows demo application

- Shows the measured values and push button information coming from the demo board

- Allows the controlling of the demo board light-emitting diodes (LED)

The demo application, called IO08USB.EXE, is located in the directory WindowsApp of the USB08 project.

# Hardware configuration

JP2

JP1_A

## Jumpers Settings

| #### You want to flash or/and debug the application ? - The jumper JP2 (near the USB cable) should be set on position Self Powered and you should use an external power supply of 8 to 20 V. - On the block JP1, all the jumpers should be set (mon08 mode). | ##### JP1 JP2 |
| --- | --- |
| #### You want to run the application in stand alone mode? - The jumper JP2 (near the USB cable) should be set on position Bus Powered. - On the block JP1, all the jumpers should be set, excepted the first one, JP1-A. | ##### JP1 JP2 |

## Connecting the Demo Board to the PC

The USB cable is fixed on the board with a cable strap. In the upstream direction (PC/host side), the USB Connector is a type A plug. Plug in this connector on the USB connector of your PC.

# Driver Installation

- After the electrical connection of the demo board, the Windows operating system recognizes the presence of a new hardware component and shows the message Found New Hardware.

- The hardware assistant tries then to find the suitable driver information for the USB08 evaluation board. Click the Next button.

- On the question Where do you want Windows to search for driver files?, mark the check box CD-ROM drives.

- The hardware assistant will find the driver information file usbio_el.inf in the root directory of the CD ROM. Confirm this selection by clicking Next.

- The Windows operating system now copies the INF file and the driver file usbio_el.sys to the appropriate Windows directories. After clicking Finish, the driver installation will be completed and the USB device will be ready for use.

NOTE: The installation using the Windows 2000 operating system requires administrator rights.

# First Steps with CodeWarrior and USB08 project

- Run the CodeWarrior IDE with the shortcut created in the program group (if not already started).

- Choose the menu File > Open and select ‘{InstallPath}\Examples\HC08\HC08 USB, then HC08 USB.mcp. Click OPEN.

## Loading the application into flash memory

- CodeWarrior opens the project that should be flashed on the JP8 board.

Note: By default, the USB communication is enabled. You can set USB_COMMUNICATION to zero in the compiler’s command line to disable it. This may be convenient if you want to debug non-USB functions such as the soft AD converter; otherwise you are likely to be stucked in interrupt driven USB routines.

- Compile the project using the menu Project > Make.

- Check the jumpers setting (See Usb08 EvaluationBoard.pdf and Jumpers settings): the board should be set in MON08 mode with external PowerSupply.

- Connect your board (connector X2) with a serial cable to the serial communication port of your computer (e.g. COM1).

- Start the debugger using the menu Project > Debug, which will flash automatically the application into the target. (See Mon08 Tips if you encounter a problem)

- The code is displayed in the Source window.

## Debugging with Mon08

- Choose the menu Run > Restart in the simulator/debugger to run your application. You will not be able to halt your program with this hardware so think about using breakpoints!

- Plug your USB connector in your PC.

- Launch IO08USB.EXE.

Be aware that the USB communication requires a constant data flow (USB is a polled bus), so your windows’ application will run into error state as as soon as you will set a breakpoint.

# Standalone Mode

Once your application has been loaded into memory, you can let it run in stand alone mode and observe the communication between your board and the PC.

- Change the Jumpers Settings

- Plug your USB connector to you PC.

- Launch IO08USB.EXE.

# Mon08 Tips

- The Mon08 Target Interface does not handle more than one breakpoint.

- Please Reset or Power on/off your board before trying to connect.

- If the password of your project does not  match the one on your derivative, try the following :

  - Please power off/on your board during 2 or 3 secondes.

  - Select Mon08 > Reconnect

  - Access the flash menu, click on select all, erase, then OK. Do not take into account the messages “Flash module Bad or protected”.

  - Select Mon08 > Setup > Password : change the password to FF FF FF FF FF FF FF FF

  - Power on/off your board.

  - Mon08 > Reconnect

  - Mon08 > Flash :You can check that the flash has been erased. Click on Load to load your .abs file into the flash. Click on OK